MS-DOS 2.0 Device Drivers

INTRODUCTION

In the  past,  DOS-device  driver  (BIOS for those who are

familiar with  CP/M)  communication  has  been  mediated  with

registers and   a  fixed-address  jump-table.   This  approach

has suffered heavily  from  the  following  two  observations:

o   The old jump-table ideas of  the  past  are  fixed  in

scope and allow no extensibility.

o   The past  device  driver  interfaces have been written

without regard for the true  power  of  the  hardware.

When   a   multitasking  system  or  interrupt  driven

hardware is installed  a  new  BIOS  must  be  written

largely from scratch.

In MSDOS  2.0, the DOS-device driver interface has changed

from the old jump-table style  to  one  in  which  the  device

drivers  are  linked  together  in  a  list.   This allows new

drivers for  optional  hardware  to  be  installed  (and  even

written) in  the  field  by other vendors or the user himself.

This flexibility is one of the major new  features  of  MS-DOS

2.0.

Each driver  in  the  chain  defines two entry points; the

strategy routine and  the  interrupt  routine.   The  2.0  DOS

does not  really make use of two entry points (it simply calls

strategy, then immediately calls interrupt).  This dual  entry

point scheme  is  designed  to facilitate future multi-tasking

versions of MS-DOS.  In multi-tasking  environments  I/O  must

be asynchronous,  to  accomplish  this  the  strategy  routine

will be called to queue  (internally)  a  request  and  return

quickly.  It  is  then  the  responsibility  of  the interrupt

routine to perform the actual I/O at interrupt time by picking

requests  off  the  internal  queue  (set  up  by the strategy

routine), and process  them.   When  a  request  is  complete,

it is  flagged  as  "done"  by the interrupt routine.  The DOS

periodically scans the  list  of  requests  looking  for  ones

flagged as  done,  and  "wakes up" the process waiting for the

completion of the request.

In order for requests to be  queued  as  above  it  is  no

longer sufficient  to pass I/O information in registers, since

many requests may be  pending  at  any  one  time.   Therefore

the new  device  interface uses data "packets" to pass request

information.  A device is called with a pointer to  a  packet,

this packet  is  linked  into  a  global  chain of all pending

I/O requests maintained by the DOS.   The  device  then  links

the packet  into  its  own  local  chain  of requests for this

particular  device.   The  device  interrupt   routine   picks

requests of  the  local  chain  for processing.  The DOS scans

the  global  chain  looking  for  completed  requests.   These

packets are  composed  of  two  pieces,  a  static piece which

has the same  format  for  all  requests  (called  the  static

request header),  which  is  followed  by information specific

to the request.  Thus packets have a variable size and format.

At this points it should be  emphasized  that  MS-DOS  2.0

does not  implement most of these features, as future versions

will.  There is no global or local queue.   Only  one  request

is pending at any one time, and the DOS waits for this current

request to be completed.  For 2.0 it  is  sufficient  for  the

strategy routine  to  simply  store  the address of the packet

at a fixed location, and for the  interrupt  routine  to  then

process  this  packet  by  doing  the  request  and returning.

Remember:  the DOS just calls the strategy  routine  and  then

immediately calls  the  interrupt  routine, it is assumed that

the request is completed when the interrupt  routine  returns.

This additional  functionality  is  defined  at  this  time so

that people will be  aware  and  thinking  about  the  future.

FORMAT OF A DEVICE DRIVER

A device  driver  is  simply  a  relocatable  memory image

with all of the code in  it  to  implement  the  device  (like

a .COM  file,  but  not ORGed at 100 Hex).  In addition it has

a special header at the front of it  which  identifies  it  as

a device,  defines  the  strategy  and interrupt entry points,

and defines various  attributes.   It  should  also  be  noted

that there are two basic types of devices.

The first  is  character devices.  These are devices which

are designed to do character  I/O  in  a  serial  manner  like

CON, AUX,  and  PRN.   These devices are named (ie.  CON, AUX,

CLOCK, etc.), and users may open channels  (FCBs)  to  do  I/O

to them.

The second  class  of  devices  is  block  devices.  These

devices are the "disk drives"  on  the  system,  they  can  do

random I/O  in  pieces  called  blocks  (usually  the physical

sector size) and  hence  the  name.   These  devices  are  not

"named" as  the  character  devices  are, and therefore cannot

be "opened" directly.   Instead  they  are  "mapped"  via  the

drive letters (A,B,C, etc.).

Block devices  also  have  units.  In other words a single

driver may be responsible for one or more  disk  drives.   For

instance block device driver ALPHA (please note that we cannot

actually  refer  to  block  devices  by  a  name!)    may   be

responsible for  drives  A,B,C  and  D, this simply means that

it has four units (0-3) defined and therefore  takes  up  four

drive letters.   Which units correspond to which drive letters

is determined by the position  of  the  driver  in  the  chain

of all  drivers:   if  driver  ALPHA is the first block driver

in the device chain, and it defines 4 units (0-3),  then  they

will be  A,B,C  and  D.   If  BETA  is the second block driver

and defines three units (0-2),  then  they  will  be  E,F  and

G and  so  on.   MS-DOS  2.0 is not limited to 16 block device

units, as  previous  versions  were.   The  theoretical  limit

is 63  (2^6  -  1),  but  it should be noted that after 26 the

drive letters get a little strange (like ] \  and  ^).   NOTE:

Character devices  cannot  define multiple units (this because

they have only one name).

Here is what that special device header looks like:

+--------------------------------------+

| DWORD Pointer to next device         |

| (Must be set to -1)                  |

+--------------------------------------+

| WORD Attributes                      |

|  Bit 15 = 1 if char device 0 if blk  |

|  if bit 15 is 1                      |

|      Bit 0 = 1 if Current sti device |

|      Bit 1 = 1 if Current sto output |

|      Bit 2 = 1 if Current NUL device |

|      Bit 3 = 1 if Current CLOCK dev  |

|      Bit 4 = 1 if SPECIAL            |

|  Bit 14 is the IOCTL bit (see below) |

|  Bit 13 is the NON IBM FORMAT bit    |

+--------------------------------------+

| WORD Pointer to Device strategy      |

|      entry point                     |

+--------------------------------------+

| WORD Pointer to Device interrupt     |

|      entry point                     |

+--------------------------------------+

| 8-BYTE character device name field   |

| Character devices set a device name  |

| For block devices the first byte is  |

| The number of units                  |

+--------------------------------------+

Note that the device entry points are  words.   They  must

be offsets  from  the  same  segment  number  used to point to

this table.  Ie.  if XXX.YYY  points  to  the  start  of  this

table, then  XXX.strategy  and  XXX.interrupt  are  the  entry

points.

A word about the Attribute  field.   This  field  is  used

most importantly  to  tell  the  system whether this device is

a block or character device (bit  15).   Most  of  other  bits

are used  to  give  selected character devices certain special

treatment (NOTE: these bits mean nothing on a  block  device).

Let's say  a  user  has  a new device driver which he wants to

be the standard input and  output.   Besides  just  installing

the driver  he  needs  to  tell  SYSINIT (and the DOS) that he

wishes his new driver to override  the  current  sti  and  sto

(the "CON"  device).   This  is  accomplished  by  setting the

attributes to the desired characteristics,  so  he  would  set

Bits 0  and 1 to 1 (note that they are separate!!).  Similarly

a  new  CLOCK  device  could  be  installed  by  setting  that

attribute, see  the  section  at  the end on the CLOCK device.

NOTE: that although there  is  a  NUL  device  attribute,  the

NUL device  cannot  be  re-assigned.   This  attribute  exists

for the DOS so that it can tell if the  NUL  device  is  being

used.

The NON  IBM  FORMAT  bit  applies  only  to block devices

and effects the operation of the  get  BPB  device  call  (see

below).

The other  bit  of  interest  is  the  IOCTL bit which has

meaning on character or block devices.   This  bit  tells  the

DOS whether  this  device  can handle control strings (via the

IOCTL system call).

If a driver cannot  process  control  strings,  it  should

initially set  this  bit  to  0.  This tells the DOS to return

an error if an attempt is made  (via  IOCTL  system  call)  to

send or  receive  control  strings  to  this device.  A device

which can process control  strings  should  initialize  it  to

1.  For  drivers  of  this  type,  the  DOS will make calls to

the IOCTL INPUT  and  OUTPUT  device  functions  to  send  and

receive   IOCTL   strings   (see  IOCTL  in  the  SYSTEM-CALLS

document).

The IOCTL functions allow data to  be  sent  and  received

by the  device  itself for its own use (to set baud rate, stop

bits, form length etc., etc.), instead of  passing  data  over

the  device  channel  as  a  normal  read  or write does.  The

interpretation of the passed information is up to the  device,

but it MUST NOT simply be treated as a normal I/O.

The SPECIAL  bit  applies  only  to  character drivers and

more particularly to  CON  drivers.   The  new  2.0  interface

is a  much  more  general  and  consistent  interface than the

old 1.25 DOS interface.  It allows for a number of  additional

features of  2.0.   It  is  also slower than 1.25 if old style

"single byte" system calls are made.  To make  most  efficient

use of  the  interface  all  applications  should  block their

I/O as much as possible.  This  means  make  one  XENIX  style

system call  to  output  X  bytes  rather  than X system calls

to output one byte each.  Also putting  a  device  channel  in

RAW   mode  (see  IOCTL)  provides  a  means  of  putting  out

characters even FASTER  than  1.25.   To  help  alleviate  the

CON output  speed  problem  for  older  programs which use the

1 - 12 system calls  to  output  large  amounts  of  data  the

SPECIAL bit  has  been implemented.  If this bit is 1 it means

the device is the  CON  output  device,  and  has  implemented

an interrupt  29  Hex  handler,  where  the  29 Hex handler is

defined as follows:

Interrupt 29h handlers

Input:

Character in AL

Function:

output the character in al to the user

screen.

Output:

None

Registers:

all registers except bx must be preserved.

No registers except for al have a known or

consistent value.

If a  character  device  implements  the  SPECIAL  bit, it

is the responsibility of the  driver  to  install  an  address

at the  correct  location in the interrupt table for interrupt

29 Hex as part of  its  INIT  code.   IMPLICATION:  There  can

be only  one  device  driver  with  the SPECIAL bit set in the

system.  There is no check to insure this state.

WARNING: THIS FEATURE WILL NOT BE SUPPORTED IN FUTURE VERSIONS

OF THE  OPERATING  SYSTEM.   IMPLICATION:  Any application

(not device driver)  which  uses  INT  29H  directly  will

not work on future versions, YOU HAVE BEEN WARNED.

In  order  to  "make"  a  device  driver  that SYSINIT can

install, a memory image or .EXE  (non-IBM  only)  format  file

must be  created  with  the  above  header  at the start.  The

link field should be  initialized  to  -1  (SYSINIT  fills  it

in).   The  attribute  field  and  entry  points  must  be set

correctly, and if the device is a character device,  the  name

field must  be  filled  in  with  the  name (if a block device

SYSINIT will fill in  the  correct  unit  count).   This  name

can be  any  8  character  "legal" file name.  In fact SYSINIT

always installs character devices at the start of  the  device

list, so  if  you  want  to  install  a new CON device all you

have to do is name it "CON".  The new  one  is  ahead  of  the

old one  in  the  list  and  thus  preempts the old one as the

search for devices stops on  the  first  match.   Be  sure  to

set the sti and sto bits on a new CON device!

NOTE:  Since  SYSINIT  may  install  the  driver anywhere, you

must be very careful about  FAR  memory  references.   You

should NOT  expect  that  your  driver will go in the same

place every time (The default BIOS  drivers  are  exempted

from this of course).

INSTALLATION OF DEVICE DRIVERS

Unlike past versions MS-DOS 2.0 allows new device  drivers

to   be   installed   dynamically   at  boot  time.   This  is

accomplished by the new SYSINIT module supplied by  Microsoft,

which reads  and  processes  the CONFIG.SYS file.  This module

is linked together with the OEM  default  BIOS  in  a  similar

manner to the way FORMAT is built.

One of  the  functions  defined  for  each device is INIT.

This routine is called once  when  the  device  is  installed,

and never  again.  The only thing returned by the init routine

is a location (DS:DX) which is a pointer  to  the  first  free

byte of  memory  after  the  device  driver, (like a terminate

and stay resident).  This pointer method can be used to "throw

away" initialization  code  that  is  only needed once, saving

on space.

Block devices are installed the same way and  also  return

a first  free  byte  pointer  as above, additional information

is also returned:

o   The number  of  units  is  returned,  this  determines

logical device  names.  If the current maximum logical

device letter is F at the time of  the  install  call,

and the init routine returns 4 as the number of units,

then they will have logical  names  G,  H,  I  and  J.

This mapping  is  determined  by  by  the  position of

the driver in the device list and the number of  units

on the  device (stored in the first byte of the device

name field).

o   A pointer to a  BPB  (Bios  Parameter  Block)  pointer

array is  also  returned.   This  will  be  similar to

the INIT table used in  previous  versions,  but  will

have more  information  in  it.   There  is  one table

for each unit defined.   These  blocks  will  be  used

to build  a  DPB  (Drive  Parameter Block) for each of

the units.  The pointer passed to  the  DOS  from  the

driver points  to  an array of n word pointers to BPBs

where n is the  number  of  units  defined.   In  this

way if  all  units  are  the same, all of the pointers

can point to the same BPB, saving space.   NOTE:  this

array must  be  protected  (below the free pointer set

by the return) since the DPB will  be  built  starting

at the  byte  pointed  to  by  the  free pointer.  The

sector size defined must be  less  than  or  equal  to

the maximum  sector  size defined at default BIOS init

time.  If it isn't the install  will  fail.   One  new

piece of DPB info set from this table will be a "media

descriptor byte".  This  byte  means  nothing  to  the

DOS, but  is  passed to devices so that they know what

form of a  DPB  the  DOS  is  currently  using  for  a

particular Drive-Unit.

Block devices  may  take  several  approaches; they may be

dumb or smart.   A  dumb  device  would  define  a  unit  (and

therefore a  DPB)  for  each possible media drive combination.

Unit 0 = drive 0 single side, unit 1 = drive  0  double  side,

etc.  For  this  approach  media  descriptor  bytes would mean

nothing.  A smart device would allow multiple media per  unit,

in this  case the BPB table returned at init must define space

large  enough  to  accommodate  the  largest  possible   media

supported.  Smart  drivers  will  use the "media byte" to pass

around info about what media is currently in  a  unit.   NOTE:

If the  DPB  is  a  "hybrid"  made  to get the right sizes, it

should give an invalid "media byte" back to the DOS.

The BOOT  (default  BIOS)  drivers  are  installed  pretty

much as  above.   The preset device list is scanned.  If block

drivers are encountered they  are  installed  as  above  (with

the exception  that  the  break is not moved since the drivers

are already resident in the  BIOS).   Note  that  the  logical

drive letters  are  assigned  in  list  order, thus the driver

which is to have logical A must  be  the  first  unit  of  the

first  block  device  in  the  list.   The  order of character

devices is also important.  There must be at least 4 character

devices defined  at  boot which must be the first four devices

(of either  type),  the  first  will  become  standard  input,

standard output,  and  standard error output.  The second will

become standard auxiliary input and  output,  the  third  will

become standard  list  output,  and  the forth will become the

date/time (CLOCK) device.  Thus  the  BIOS  device  list  must

look like this:

->CON->AUX->PRN->CLOCK->any other block or character devices

THE DRIVER

A device driver will define the following functions:

Command   Function

Code

0      INIT

1      MEDIA CHECK (Block only, NOP for character)

2      BUILD BPB      "    "     "    "   "

3      IOCTL INPUT (Only called if device has IOCTL)

4      INPUT (read)

5      NON-DESTRUCTIVE INPUT NO WAIT (Char devs only)

6      INPUT STATUS                    "     "    "

7      INPUT FLUSH                     "     "    "

8      OUTPUT (write)

9      OUTPUT (Write) with verify

10      OUTPUT STATUS                   "     "    "

11      OUTPUT FLUSH                    "     "    "

12      IOCTL OUTPUT (Only called if device has IOCTL)

As mentioned before, the first entry point is the strategy

routine which  is called with a pointer to a data block.  This

call does not perform the request, all it  does  is  queue  it

(save the  data  block  pointer).   The second interrupt entry

point is called immediately  after  the  strategy  call.   The

"interrupt" routine  is called with no parameters, its primary

function is to perform  the  operation  based  on  the  queued

data block and set up any returns.

The "BUILD  BPB"  and  "MEDIA  CHECK"  are the interesting

new ones, these are explained by  examining  the  sequence  of

events in the DOS which occurs when a drive access call (other

than read or write) is made:

I.  Turn drive letter  into  DPB  pointer  by  looking

for DPB with correct driver-unit number.

II. Call device  driver  and  request  media check for

Drive-Unit.   DOS   passes   its   current   Media

descriptor byte (from DPB).  Call returns:

Media Not Changed

Media Changed

Not Sure

Error

Error - If an error occurs the error  code  should

be set accordingly.

Media Not  changed  -  Current  DPB and media byte

are OK, done.

Media Changed - Current DPB and media  are  wrong,

invalidate any  buffers  for  this  unit,  and

goto III.

Not Sure - If there are  dirty  buffers  for  this

unit, assume  DPB  and  media  byte are OK and

done.  If nothing dirty, assume media changed,

invalidate any  buffers  for  unit,  and  goto

III.

NOTE:  If a hybrid  DPB  was  built  at  init  and

an invalid  Media  byte  was  set,  the driver

should return media changed when this  invalid

media byte is encountered.

III. Call device  driver  to build BPB with media byte

and buffer.

What the  driver  must  do  at  step  III is determine the

correct media that is currently in  the  unit,  and  return  a

pointer to  a  BPB table (same as for the install call).  This

table will be used as at init  to  build  a  correct  DPB  for

the unit  If the determined media descriptor byte in the table

turns out to be the same as the one passed in,  then  the  DOS

will not  build  a new table, but rather just use the old one.

Therefore in this case the driver doesn't  have  to  correctly

fill in the other entries if desired.

The build  BPB  call  also  gets a pointer to a one sector

buffer.  What this buffer contains is determined  by  the  NON

IBM FORMAT  bit  in  the  attribute field.  If the bit is zero

(device is IBM format compatible)  then  the  buffer  contains

the first  sector  of  the  first  FAT,  in particular the FAT

ID byte is the first byte  of  this  buffer.   NOTE:  It  must

be true  that  the  BPB is the same, as far as location of the

FAT is concerned, for all possible  media.   This  is  because

this first  FAT  sector  must  be  read  BEFORE the actual BPB

is returned.  If the NON  IBM  FORMAT  bit  is  set  then  the

pointer points  to  one  sector  of scratch space which may be

used for anything.

CALL FORMAT

When the  DOS calls a device driver to perform a finction,

it passes a  structure  (Drive  Request  Structure)  in  ES:BX

to perform  operations  and  does  a long call to the driver's

strategy entry point.  This structure is a fixed length header

(Static Request  Header)  followed  by  data  pertinent to the

operation  being  performed.   NOTE:   It   is   the   drivers

responsibility to preserve machine state.

STATIC REQUEST HEADER ->

+-----------------------------+

| BYTE length of record       |

|  Length in bytes of this    |

|  Drive Request Structure    |

+-----------------------------+

| BYTE unit code              |

|  The subunit the operation  |

|  is for (minor device)      |

|  (no meaning on character   |

|   devices)                  |

+-----------------------------+

| BYTE command code           |

+-----------------------------+

| WORD Status                 |

+-----------------------------+

| 8 bytes reserved here for   |

| two DWORD links. One will   |

| be a link for the DOS queue |

| The other for the device    |

| queue                       |

+-----------------------------+

STATUS WORD

15  14 13 12 11 10  9   8   7  6  5  4  3  2  1  0

+---+---+--+--+--+--+---+---+--+--+--+--+--+--+--+--+

| E |               | B | D |                       |

| R |   RESERVED    | U | O | ERROR CODE (bit 15 on)|

| R |               | I | N |                       |

+---+---+--+--+--+--+---+---+--+--+--+--+--+--+--+--+

The status  word is zero on entry and is set by the driver

interrupt routine on return.

Bit 8 is the done bit, it means the operation is complete.

For the  moment  the Driver just sets it to one when it exits,

in the future this will be set by  the  interrupt  routine  to

tell the DOS the operation is complete.

Bit 15 is the error bit, if it  is  set  then  the  low  8

bits indicate the error:

0 Write Protect violation

(NEW)  1 Unknown Unit

2 Drive not ready

(NEW)  3 Unknown command

4 CRC error

(NEW)  5 Bad Drive Request Structure length

6 Seek error

(NEW)  7 Unknown media

8 Sector not found

(NEW)  9 Printer out of paper

A Write Fault

(NEW)  B Read Fault

C General Failure

Bit 9 is the busy bit which is set only by status  calls  (see

STATUS CALL below).

Here is the data block format for each function:

READ or WRITE - ES:BX (Including IOCTL) ->

+------------------------------------+

| 13-BYTE  Static Request Header     |

+------------------------------------+

| BYTE Media descriptor  from DPB    |

+------------------------------------+

| DWORD transfer address             |

+------------------------------------+

| WORD byte/sector Count             |

---+------------------------------------+---

| WORD starting sector number        |

|  (ignored on Char Devs)            |

+------------------------------------+

In addition to setting the status word,  the  driver  must

set the  Sector  count  to  the  actual  number of sectors (or

bytes) transferred.  NOTE: No  error  check  is  performed  on

an IOCTL I/O call, driver MUST correctly set the return sector

(byte) count  to  the  actual  number  of  bytes  transferred,

however.

NOTE: THE FOLLOWING APPLIES TO BLOCK DEVICE DRIVERS.

Under certain  circumstances  the  BIOS  may  be  asked to

do a write operation of 64K bytes which seems to  be  a  "wrap

around" of  the transfer address in the BIOS I/O packet.  This

arises due to an optimization  added  to  the  write  code  in

MS-DOS.  It will only manifest on user WRITEs which are within

a sector size of 64K bytes on files which are  "growing"  past

the current  EOF.   IT  IS  ALLOWABLE  FOR  THE BIOS TO IGNORE

THE BALANCE OF  THE  WRITE  WHICH  "WRAPS  AROUND"  IF  IT  SO

CHOOSES.   For  instance  a  WRITE  of  10000H  bytes worth of

sectors with a transfer address  of  XXX:1  could  ignore  the

last two bytes (remember that a user program can never request

an I/O of more than FFFFH bytes and cannot wrap  around  (even

to 0)  in  his  transfer segment, so in this case the last two

bytes can be ignored).

NON DESRUCTIVE READ NO WAIT - ES:BX ->

+------------------------------------+

| 13-BYTE Static Request Header      |

+------------------------------------+

| BYTE read from device              |

+------------------------------------+

This call is analogous to the console  input  status  call

on MS-DOS  1.25.   If  the  character  device returns Busy bit

= 0 (characters in  buffer),  then  the  next  character  that

would be  read  is  returned.   This  character is NOT removed

from the input buffer (hence the term Non  Destructive  Read).

In essence  this  call  allows the DOS to look ahead one input

character.

MEDIA CHECK - ES:BX ->

+------------------------------------+

| 13-BYTE  Static Request Header     |

+------------------------------------+

| BYTE Media Descriptor from DPB     |

+------------------------------------+

| BYTE returned                      |

+------------------------------------+

In addition to setting status word, driver  must  set  the

return byte.

Return Byte :

-1 Media has been changed

0 Don't know if media has been changed

1 Media has not been changed

If the driver can return -1 or 1 (by  having  a  door-lock

or other  interlock  mechanism)  the  performance of MSDOS 2.0

is enhanced as the DOS  need  not  reread  the  FAT  for  each

directory access.

BUILD BPB - ES:BX ->

+------------------------------------+

| 13-BYTE Static Request Header      |

+------------------------------------+

| BYTE Media Descriptor from DPB     |

+------------------------------------+

| DWORD Transfer Address             |

| (points to one sectors worth of    |

|  scratch space or first sector     |

|  of FAT depending on the value     |

|  of the NON IBM FORMAT bit)        |

+------------------------------------+

| DWORD Pointer to BPB               |

+------------------------------------+

If the NON IBM FORMAT bit  of  the  device  is  set,  then

the DWORD  Transfer  Address  points  to  a  one sector buffer

which can be used for any purpose.   If  the  NON  IBM  FORMAT

bit is  0,  then  this buffer contains the first sector of the

FAT; in this case the driver must not alter this buffer  (this

mode is  useful  if  all  that  is  desired is to read the FAT

ID byte).

If IBM compatible format  is  used  (NON  IBM  FORMAT  BIT

= 0),  then it must be true that the first sector of the first

FAT is located at the  same  sector  on  all  possible  media.

This is  because  the FAT sector will be read BEFORE the media

is actually determined.

In addition to setting status word, driver  must  set  the

Pointer to the BPB on return.

In order to allow for many different  OEMs  to  read  each

other's disks,  the  following  standard  is  suggested:   The

information relating to the BPB  for  a  particular  piece  of

media   is  kept  in  the  boot  sector  for  the  media.   In

particular, the format of the boot sector is:

+------------------------------------+

| 3 BYTE near JUMP to boot code      |

+------------------------------------+

| 8 BYTES OEM name and version       |

---+------------------------------------+---

B  | WORD bytes per sector              |

P  +------------------------------------+

B  | BYTE sectors per allocation unit   |

+------------------------------------+

|  | WORD reserved sectors              |

V  +------------------------------------+

| BYTE number of FATs                |

+------------------------------------+

| WORD number of root dir entries    |

+------------------------------------+

| WORD number of sectors in logical  |

^  | image                              |

|  +------------------------------------+

B  | BYTE media descriptor              |

P  +------------------------------------+

B  | WORD number of FAT sectors         |

---+------------------------------------+---

| WORD sectors per track             |

+------------------------------------+

| WORD number of heads               |

+------------------------------------+

| WORD number of hidden sectors      |

+------------------------------------+

The three  words  at the end are optional, the DOS doesn't

care about them (since they are not part of  the  BPB).   They

are intended  to  help the BIOS understand the media.  Sectors

per track may be redundant (could be figured  out  from  total

size of  the  disk).  Number of heads is useful for supporting

different  multi-head  drives  which  have  the  same  storage

capacity, but  a  different  number  of  surfaces.   Number of

hidden sectors is useful  for  supporting  drive  partitioning

schemes.

Currently, the media  descriptor  byte  has  been  defined

for a small range of media:

5 1/4" diskettes:

Flag bits:

01h - on -> 2 double sided

All other bits must be on.

8" disks:

FEh - IBM  3740 format, singled-sided, single-density,

128 bytes per sector,  soft  sectored,  4  sectors

per allocation  unit,  1  reserved sector, 2 FATs,

68 directory entries, 77*26 sectors

FDh -   8"    IBM    3740    format,    singled-sided,

single-density,   128   bytes   per  sector,  soft

sectored,  4  sectors  per  allocation   unit,   4

reserved sectors,  2  FATs,  68 directory entries,

77*26 sectors

FEh - 8"  Double-sided,  double-density,  1024   bytes

per sector, soft sectored, 1 sector per allocation

unit, 1 reserved sector,  2  FATs,  192  directory

entries, 77*8*2 sectors

STATUS Calls - ES:BX ->

+------------------------------------+

| 13-BYTE Static Request Header      |

+------------------------------------+

All driver must do is  set  status  word  accordingly  and

set the busy bit as follows:

o   For output on  character  devices:   If  it  is  1  on

return, a  write  request  (if  made)  would  wait for

completion of a current request.  If it  is  0,  there

is no  current  request  and a write request (if made)

would start immediately.

o   For input on character devices with a buffer a  return

of 1  means,  a  read  request  (if  made) would go to

the physical device.  If  it  is  0  on  return,  then

there are  characters  in  the  devices  buffer  and a

read would return  quickly,  it  also  indicates  that

the user  has  typed  something.   The DOS assumes all

character devices have an  input  type  ahead  buffer.

Devices which  don't  have  them  should always return

busy = 0 so  that  the  DOS  won't  hang  waiting  for

something to  get  into  a buffer which doesn't exist.

FLUSH Calls - ES:BX ->

+------------------------------------+

| 13-BYTE Static Request Header      |

+------------------------------------+

This  call  tells  the  driver  to  flush  (terminate) all

pending requests  that  it  has knowledge of.  Its primary use

is to flush the input queue on character devices.

INIT - ES:BX ->

+------------------------------------+

| 13-BYTE Static Request Header      |

+------------------------------------+

| BYTE # of units                    |

+------------------------------------+

| DWORD Break Address                |

---+------------------------------------+---

| DWORD Pointer to BPB array         |

| (not set by Character devices)     |

+------------------------------------+

The number of units, break address, and  BPB  pointer  are

set by the driver.

FORMAT OF BPB (Bios Parameter Block) -

+------------------------------------+

| WORD Sector size in Bytes          |

|    Must be at least 32             |

+------------------------------------+

| BYTE Sectors/Allocation unit       |

|    Must be a power of 2            |

+------------------------------------+

| WORD Number of reserved sectors    |

|        May be zero                 |

+------------------------------------+

| BYTE Number of FATS                |

+------------------------------------+

| WORD Number of directory entries   |

+------------------------------------+

| WORD Total number of sectors       |

+------------------------------------+

| BYTE Media descriptor              |

+------------------------------------+

| WORD Number of sectors occupied by |

|      FAT                           |

+------------------------------------+

THE CLOCK DEVICE

One of  the  most  popular add on boards seems to be "Real

Time CLOCK Boards".  To allow these boards  to  be  integrated

into the  system  for TIME and DATE, there is a special device

(determined by the attribute word) which is the CLOCK  device.

In all  respects  this  device  defines and performs functions

like any other character device (most functions will  be  "set

done bit,  reset  error  bit,  return).   When a read or write

to this device occurs, exactly 6 bytes are transferred.   This

I/O can be thought of as transferring 3 words which correspond

exactly to the values of AX, CX and  DX  which  were  used  in

the old  1.25  DOS  date  and  time  routines.  Thus the first

two bytes are a word which is the count of days since  1-1-80.

The  third  byte  is  minutes,  the  fourth  hours,  the fifth

hundredths of seconds, and the  sixth  seconds.   Reading  the

CLOCK device  gets  the  date and time, writing to it sets the

date and time.

#######################################################################